Response memo: technical review of 30 October 2026

Manuscript: paper/EV-PIL-manuscript.docx (25 pages), rendered PDF alongside. Status per item: corrected, supported but unchanged, or not yet verifiable.

| Item | Status | What changed and why |
| --- | --- | --- |
| R1 DP interpretation | Corrected | Overclaims fixed in I, II, V-B, IX-A, X and README ("that much to gain", "DP bound", "within ... of the optimum"). V-B now states the approximations (single SOC state, exogenous RC branch, fixed ceiling) and that the gap does not certify the attainable saving. Optional grid check run: reference unchanged on SOC grids of 81 to 321 points and 6 to 77 action combinations. |
| R2 Three-seed traceability | Corrected | Records were already public on master; the plan named a combined file that was never written. Erratum added to the plan; docs/reproducibility-manifest.md gives paths, checksums and one command per table; run ledger (Table 2, CSV per run) added. Statistical unit stated in V-F. One script stamped the current commit into the manifest on re-run; fixed. |
| R3 Measured vs model energy | Corrected | Three labels defined in III (target-measured, PIL-evaluated, host-simulated) and applied to every result, table and caption. Abstract, I, V-E and README now say the EMS runs at 10 Hz above a 100 Hz speed loop. |
| R4 Layout and placeholders | Corrected; affiliation not yet verifiable | Cause of the Table 8 / Fig. 6 overlap removed: page-wide items no longer float. Mock note, dates, DOI, volume line and placeholder bios removed; AI-use statement and public data statement added. Every page rendered through Word and inspected. Affiliation left marked for the authors, as instructed. |
| R5 Audit framing | Corrected | Table 6 sets the audit in the four requested groups with path, result and coverage; tested parts separated from recommended ones. States that beating a feasible reference is a reason to investigate, not proof. Carried into abstract, I, IX-B and X. |
| R6 Model-validity disclosures | Supported; local changes | Existing disclosures kept. Table 7 summarises validity and its consequence for reported figures; abstract, limitations and conclusion now say absolute Wh/km belong to the model (13% to 17% transfer error). Test trips stay sealed. |
| R7 to R10 (P1) | Corrected (R10 partly) | R7 detector claims scoped; alarm onsets and time flagged separated; interpolation not treated as a 100 Hz sensor; removed from abstract. R8 Table 13 sets baseline, low-pass and Kalman side by side; recovery credited to filtering. R9 ledger, deployed-seed row, SAC-only scope. R10 VII-E/F shortened, VI to VIII marked supporting, authoritative version named; forecasting kept. |

New experiments: one host-only check (DP grid sensitivity, optional in R1). No board run was needed; the review asks for claim accuracy, provenance and presentation, and the existing records answered it. No result changed sign or value.

Still with the authors: confirmed affiliations; biographies when the venue is fixed; review of the AI-use statement against the venue's policy.